Lab01 Report

- I compensate my probes in this order; First I plugged my probe to the digital oscilloscope and set to auto set After that I observe that may probe is over compensated and in order to fix it I used with the black tool(control stick) which comes with the probe. Finally I obtained square wave graphic

- I prepared my system which means signal generator is giving 5 Vp-p sinusoidal signal with a 1 kHz frequency. After that First I used positive edge triggering and I obtained fig.1

fig.1

After obtaining that I changed to negative edge triggering and I obtained fig.2

fig.2

and after observing two of them I conclude that changing negative /positive trigger mode is chancing how oscilloscope is trigged in the way it is fallin or rising at that point and because of that we have diplayed slightly different graphics

- I prepare my setup as giving 1Vp-p with 2 kHz frequency of triangular wave. After that I change the trigger level knob on the oscilloscope and I observe that when I am raising the trigger mode oscilloscope cannot display the graph because it is not reaching the trigger level  and when I am decreasing the level it is diplaying the graph as it should be.

- I prepare my setup as giving 1Vp-p with 5 kHz frequency of square wave. Then I select all of the acquisition modes which are sample, peak detect and average modes. First I used  Sample mode which showed me waveform points (fig.3). After I used peak detect mode which shows  and I obtained fig.4 which showed me the stored  peak point on the normal graph. And finally I used averge mode and I obtained fig.5 which shows me more stable graph because it stroes the average points.

fig.3

fig.4

fig.5

- I prepare my setup as giving 2Vp-p with 1 kHz frequency of sinusoidol wave and also I apple DC-Offset of 1 V. Than I used DC coupling and obtained fig.6  after I switch the AC coupling I obtained fig. 7. I observe that In DC copling both of the current types is shoen in the graph but in the AC coupling only AC is detecting DC is not.

Fig.6

fig.7

- I setup the board as it given in the assignment and I prepare my system as giving 2Vp-p with 1 kHz frequency of sinusoidol wave with DC-Offset of 0. I obtained fig.8 and I measuered the time delay 36µ second and phase difference is 12 decrease. After I change to 100 Hz I obtained fig.9 and  1.6 ms time delay and  57 degrease of phase diiference. I observe that this circuit failed at filtering high frequency signal but do well at low freuency signals.

fig.8fig.9

Şevki Gavrem Kulkuloğlu

21601793 section 2