- Design Methodology

I started my VGA controller project with counting the pixels on screen first horizontally and when a row is done, I passed new row in downwards until every row is done. While counting I determine the pixel locations according to screens resolution which means its horizontal and vertical porches and sync pulses, so pixel locations will be in the active (displayed) screen. After I mapped out the screen, only thing remained was the color of the screen and its attachment to switches, so I assign total 12 switches which goes like 4 switches to red, 4 switches to green, 4 switches to blue. I have also assigned a button for disabling the view, simply turning off the VGA port output.

- In my preliminary lab report I stated that I want to make the initial color of the screen white. At first, I thought because at the beginning switches are ‘0’ it made output conflicts but afterwards I got the idea that using switches as inverted way. After inverted them, it worked as I planned.

- As I worked on my project and learnt VGA more my modules and especially my design has been changed compared to my preliminary lab report.

Results

RTL Schematic

Like it is seen in the figure above my idea at the preliminary report was substantially right but I need to make some changes in the modules and their designs.

Sync module

Paint module

- Screen shot of the display and Basys3

Because I want to obtain white screen at opening, I used switches as inverted. Most significant 4 switches assigned for red, 4 center switches for assigned green and least significant 4 switches for assigned blue.

Red = “1111”, Green = “1111”, Blue = “1111”

Red = “1111”, Green = “0000”, Blue = “0000”

Red = “0000”, Green = “0000”, Blue = “1111”

Red = “0000”, Green = “1111”, Blue = “0000”

Red = “1111”, Green = “1111”, Blue = “0000”

Red = “1111”, Green = “0000”, Blue = “1111”

- Conculusion

At first I was thinking take the resolution as 1280x1024 and I did, but I did not obtain what I expected for display, it was like 2/3 part of the screen was active and rest was black and I could not change it no matter what I did with those spects, I searched on internet about spects of the screen I used and I found that it was 1440x900. It is a rare resoulution actually. I change my code respect to those spects and afterward I finally got what I wanted from the display.

- Appendices

Top Module:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity Top_Module is

Port ( clk       : in std_logic;

clear_btn : in std_logic;

Switches  :  in std_logic_vector(11 downto 0);

H_sync    : out std_logic;

V_sync    : out std_logic;

red       : out std_logic_vector(3 downto 0);

green     : out std_logic_vector(3 downto 0);

blue      : out std_logic_vector(3 downto 0));

end Top_Module;

architecture Behavioral of Top_Module is

component Synch is

Port ( clk100Mhz  : in std_logic;

clear      : in std_logic;

H_sync     : out std_logic;

V_sync     : out std_logic;

D_enable   : out std_logic;

Location_H : out std_logic_vector(10 downto 0);

Location_V : out std_logic_vector(10 downto 0));

end component;

component Paint is

Port (D_enable   : in std_logic;

Location_H :  in std_logic_vector(10 downto 0);

Location_V :  in std_logic_vector(10 downto 0);

Switches   :  in std_logic_vector(11 downto 0);

red        :  out std_logic_vector(3 downto 0);

blue       :  out std_logic_vector(3 downto 0);

green      :  out std_logic_vector(3 downto 0));

end component;

signal D_enable : std_logic;

signal clr : std_logic;

signal Location_H : std_logic_vector(10 downto 0);

signal Location_V : std_logic_vector(10 downto 0);

begin

clr <= clear_btn;

c2 : Synch port map(clk, clr, H_sync, V_sync, D_enable, Location_H, Location_V);

c3 : Paint port map( D_enable, Location_H, Location_V,Switches, red, green, blue);

end Behavioral;

Sync:

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

use IEEE.STD_LOGIC_ARITH.ALL;

entity Synch is

Port ( clk100Mhz    : in std_logic;

clear        : in std_logic;

D_enable     : out std_logic;

Location_H   : out std_logic_vector(10 downto 0);

Location_V   : out std_logic_vector(10 downto 0);

H_sync       : out std_logic;

V_sync       : out std_logic);

end Synch;

architecture Behavioral of Synch is

constant Pixels_H    : integer := 1904;

constant Pixels_V    : integer := 932;

constant hf_porch    : integer := 1824;

constant vf_porch    : integer := 931;

constant hb_porch    : integer := 232;

constant vb_porch    : integer := 28;

constant hs_pulse    : integer := 152;

constant vs_pulse    : integer := 3;

signal V_enable      : std_logic;

signal Horizontal_Counter         : integer range 0 to 1904;

signal Vertical_Counter           : integer range 0 to 932;

begin

process(clk100Mhz, clear)

begin

if clear = '1' then

Horizontal_Counter <= 0;

elsif(clk100Mhz'event and clk100Mhz = '1') then

if Horizontal_Counter = Pixels_H - 1 then

Horizontal_Counter <= 0;

V_enable <= '1';

else

Horizontal_Counter <= Horizontal_Counter + 1;

V_enable <= '0';

end if;

end if;

end process;

H_sync <= '0' when Horizontal_Counter < 112 else '1';

process(clk100Mhz, clear)

begin

if clear = '1' then

Vertical_Counter <= 0;

elsif(clk100Mhz'event and clk100Mhz = '1' and V_enable = '1') then

if Vertical_Counter = Pixels_V - 1 then

Vertical_Counter <= 0;

else

Vertical_Counter <= Vertical_Counter + 1;

end if;

end if;

end process;

V_sync <= '0' when Vertical_Counter < 3 else '1';

D_enable <= '1' when (((Horizontal_Counter < hf_porch) and (Horizontal_Counter >= (hb_porch + hs_pulse))) and ((Vertical_Counter < vf_porch) and (Vertical_Counter >= (vb_porch + vs_pulse)))) else '0';

Location_H <= conv_std_logic_vector(Horizontal_Counter, 11);

Location_V <= conv_std_logic_vector(Vertical_Counter, 11);

end Behavioral;

Paint

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity Paint is

Port (D_enable    : in std_logic;

Location_H  : in std_logic_vector(10 downto 0);

Location_V  : in std_logic_vector(10 downto 0);

Switches    : in std_logic_vector(11 downto 0);

red         : out std_logic_vector(3 downto 0);

blue        : out std_logic_vector(3 downto 0);

green       : out std_logic_vector(3 downto 0));

end Paint;

architecture Behavioral of Paint is

begin

process(D_enable,Location_H,Location_V)

begin

red   <= "0000";

blue  <= "0000";

green <= "0000";

if D_enable = '1' then

if Location_V > 30 and Location_V < 932 and Location_H > 384 and Location_H < 1904 then

red   <= not Switches(11 downto 8);

green <= not Switches(7 downto 4);

blue  <= not Switches(3 downto 0);

end if;

end if;

end process;

end Behavioral;